EMBEDDED SYSTEM

### Experiment 1 – Flashing of LED using AT89C51 using Proteus

OBJECTIVES:

- To understand LED interfacing with the AT89C51 microcontroller.

- To write an assembly language program for LED flashing.

- To simulate and verify the LED flashing operation using Proteus.

PROGRAM:

PROGRAM

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT: